Muhammad Ahmad

21L-5617 BS(DS)7A

In-Lab Statement 1: Analyzing HTTP Protocol

Q1 : The four protocols that appear in the unfiltered packet-listing window are TCP, HTTP, SNMP, and DNS.

Q2: The time taken from when the HTTP GET message was sent until the HTTP OK reply was received is approximately 0.024 seconds.

Q3: The second GET request was not successful, as indicated by the 404 Not Found status in the response packet.

Q4: Both the browser and the server are running on HTTP/1.1.

Q5: browser will request content in U.S. English first, but will also accept content in general English if U.S. English is not available

Q6:

 IP address of my computer: 192.168.1.102

 IP address of the gaia.cs.umass.edu server: 128.119.245.12

Q7:

 MAC address of my computer (Source): 00:08:74:4f:36:23 (Dell_4f:36:23)

 MAC address of the destination  00:06:25:da:af:73 (LinksysGroup_da:af:73)

Q8:

- Source Port: 4127 (This is the port of my computer sending the request).

- Destination Port: 80 (This is the port on the server).

### Port 80:

Port 80 is the default port for the HTTP protocol, which is used for standard web communication. When your browser requests a webpage, it typically sends the request to the server's port 80.

Q9: The status code returned from the server to my browser is 200,

Q10: The HTML file that you are retrieving was last modified on the server on Tue, 23 Sep 2003 05:29:00 GMT

Q11: The total packet content being returned to my browser is 73 bytes

The HTTP CONDITIONAL GET/response interaction

Q1: No, there is no "If-Modified-Since" line in the first HTTP GET request from the browser to the server.

Q2: Yes, the server explicitly returned the contents of the file, as indicated by the presence of the text "Congratulations again! Now you've downloaded the file lab2-2.html. <br>\n …." in the Packet Bytes Window.

Q3: Yes, the second HTTP GET request includes "If-Modified-Since: Tue, 23 Sep 2003 05:35:00 GMT." This means the browser requests the resource only if it has been modified since that date

Q4:

The HTTP status code returned from the server in response to the second HTTP GET is 304 Not Modified.

### Explanation:

The server did not explicitly return the contents of the file, as indicated by the 304 Not Modified status. This means the resource has not changed since the date specified in the If-Modified-Since header, so the browser should use its cached version instead.

In-Lab Statement 2 : Analyzing HTTP Protocol

Q1: browser sent a total of one HTTP GET request messages.

Q2: The packet number containing the GET message for The Bill of Rights is 8.

Q3: The packet number that contains the status code and phrase associated with the response to the HTTP GET request is 14. This packet shows the response with HTTP/1.1 200 OK.

Q4: The status code in the response is 200, and the phrase is "OK."

Q5:

- Total Content Length: 4500 bytes

- Size of Each Segment: 436 bytes

### Number of Segments:

Number of Segments=4500/436≈10.3

Total Segments Needed: 11

In-Lab Statement 3: Trick Question

Length of the text for The Bill of Rights: 4500 bytes.

Justification: The Response Packet Size of 490 bytes includes headers and part of the content, not the full text.

Explanation:

- Content-Length is 4500 bytes.

- First packet: 490 bytes (headers + some content).

- Remaining content: 4500 - 490 = 4010 bytes.

- Each subsequent packet carries 436 bytes.

- Number of additional packets needed: 4010 / 436 ≈ 9.2, so 10 packets.

- Total segments: 1 (490 bytes) + 10 (436 bytes each) = 11 segments to reach 4500 bytes.