Muhammad Ahmad

21L-5617 BS(DS)7A

Question 1

1:

The 226 Transfer Complete response indicates a successful file transfer over port 21 (FTP control).

Port 20 is used for FTP data transfer.

2:

Here is the information for the 19 packets obtained by applying the ftp || ftp-data filter

 Packet 89: The server at IP: 195.89.6.167 (Port: 21) responds to the user's request with response code 220.

 Packet 94: The client at IP: 192.168.1.2 (Port: 16340) sends a login request using the command USER anonymous.

 Packet 96: The server at IP: 195.89.6.167 (Port: 21) replies with code 331, requesting a password.

 Packet 99: The client at IP: 192.168.1.2 (Port: 16340) attempts to authenticate by sending the PASS command.

 Packet 100: The server at IP: 195.89.6.167 (Port: 21) replies with response code 230, indicating successful login.

 Packet 104: The client at IP: 192.168.1.2 (Port: 16340) requests data transfer from the server.

 Packet 105: The server at IP: 195.89.6.167 (Port: 21) responds with response code 200.

 Packet 106: The client at IP: 192.168.1.2 (Port: 16340) sends the command NLST to the server.

 Packet 107: The server at IP: 195.89.6.167 (Port: 21) responds with code 150, indicating it is ready to open a data connection.

 Packet 125: The server at IP: 195.89.6.167 (Port: 21) completes the data transfer and replies with response code 226.

 Packet 127: Data is transferred between the server (IP: 195.89.6.167, Port: 20) and the client.

 Packet 151: The client at IP: 192.168.1.2 (Port: 16340) sends another request to the server.

 Packet 152: The server at IP: 195.89.6.167 (Port: 21) replies with response code 200.

 Packet 153: The client at IP: 192.168.1.2 (Port: 16340) sends another command to the server.

 Packet 155: The server at IP: 195.89.6.167 (Port: 21) replies with response code 122.

 Packet 160: The server at IP: 195.89.6.167 (Port: 21) completes the transfer and responds with code 226.

 Packet 161: FTP data is exchanged between the server (IP: 195.89.6.167, Port: 20) and the client.

 Packet 173: The client at IP: 192.168.1.2 (Port: 16340) issues the QUIT command to terminate the session.

 Packet 175: The server at IP: 195.89.6.167 (Port: 21) replies with response code 221, confirming the session closure.

Question2

1:

No, ICMP messages are not sent over UDP or TCP; they are sent directly over IP.

2:

 The link-layer (Ethernet) address of the source host is 60:67:20:55:7b

.

 The link-layer (Ethernet) address of the destination host is c0:4a:00:87:05

3:

Type: 8 (Echo (ping) request)

Type: 3 (Destination unreachable)

Type: 0 (Echo (ping) reply)

4:

Total ICMP Requests Sent: 4 Echo Requests

5:

 My laptop Host IP Address: 192.168.33.110

 Destination Host IP Address: 172.217.27.36

Q6 :

ICMP works at the network layer to help with network issues and error messages.ICMP doesn't need port numbers because it's not meant for application communication.

Q7:

An ICMP Echo Request has a Type value of 8, while an ICMP Echo Reply has a Type value of 0.

### 8:

### ICMP Echo Request Packet Details

- ICMP Type: 8 (Echo Request)

- ICMP Code: 0

### Additional Fields in the ICMP Packet

### Checksum:

  - Value: 0x4d38

  - Status: Good (indicating no errors)

  - Size: 2 bytes

- Identifier:

  - Big Endian (BE): 1 (0x0001)

  - Little Endian (LE): 256 (0x0100)

  - Size: 2 bytes

- Sequence Number:

  - Big Endian (BE): 35 (0x0023)

  - Little Endian (LE): 8960 (0x2300)

  - Size: 2 bytes

### Sizes

- Checksum Size: 2 bytes

- Identifier Size: 2 bytes

- Sequence Number Size: 2 bytes

### 9:

### ICMP Echo Reply Packet Details

- ICMP Type: 0 (Echo Reply)

- ICMP Code: 0

### Additional Fields in the ICMP Packet

- Checksum:

  - Value: 0x5539

  - Status: Good (indicating no errors)

  - Size: 2 bytes

- Identifier:

  - Big Endian (BE): 1 (0x0001)

  - Little Endian (LE): 256 (0x0100)

  - Size: 2 bytes

- Sequence Number:

  - Big Endian (BE): 34 (0x0022)

  - Little Endian (LE): 8704 (0x2200)

  - Size: 2 bytes

- Response Time: 98.363 ms

### Summary of Field Sizes

- Checksum Size: 2 bytes

- Identifier Size: 2 bytes

- Sequence Number Size: 2 bytes

### 10:

### Packet No. 56 ICMP Details

- ICMP Type: 3 (Destination Unreachable)

- ICMP Code: 3 (Port Unreachable)

- IP and TCP Headers: Included to identify the original packet causing the error.

- Header Depiction: IP header shows source/destination addresses; TCP header shows segment and ports involved.